AI/ML Functional Requirements
AI-Powered Predictive Maintenance System

1. Purpose
This document defines the Artificial Intelligence and Machine Learning Functional Requirements for the AI-Powered Predictive Maintenance System.
The AI/ML component is responsible for processing time-series equipment sensor data, detecting abnormal machine behavior, predicting potential failures, estimating Remaining Useful Life (RUL), classifying machine health severity, and providing explainable predictions.
The requirements define what the AI/ML component shall do and provide a foundation for the system design, implementation, testing, and integration with the FastAPI backend and Streamlit application.

2. AI/ML Scope
The AI/ML pipeline shall support the following major capabilities:
Sensor data validation
Time-series preprocessing
Feature engineering and extraction
Anomaly detection
Remaining Useful Life prediction
RUL uncertainty estimation
Machine health severity classification
Explainable AI
Configurable prediction thresholds
Model inference
Model evaluation
Model versioning and experiment tracking
Data drift monitoring
Model retraining
Prediction and model logging

3. Functional Requirements
FR-AI-01 — Sensor Data Validation
Description:
The system shall validate incoming sensor data before passing it to the AI/ML processing pipeline.
The validation process shall check:
Required sensor fields are available.
Data types are valid.
Missing values are identified.
Sensor values are within predefined operating ranges.
Timestamps are valid and properly ordered.
Invalid or corrupted records are flagged.
Input:
Vibration, temperature, pressure, timestamp, machine ID, and other available sensor attributes.
Output:
Validated sensor data or validation errors/flags.
Priority: High

FR-AI-02 — Time-Series Data Preprocessing
Description:
The system shall preprocess validated sensor data before applying machine learning or deep learning models.
The preprocessing pipeline shall support operations including:
Missing-value handling
Noise reduction where required
Normalization or standardization
Time-series windowing
Resampling where necessary
Data transformation
For vibration signals, the system shall support appropriate signal-processing techniques such as Fast Fourier Transform (FFT).
Input:
Validated sensor time-series data.
Output:
Preprocessed time-series data ready for feature engineering and model inference.
Priority: High

FR-AI-03 — Feature Engineering and Extraction
Description:
The system shall generate relevant features from time-series sensor data to support machine health analysis and predictive modeling.
The feature engineering process may include:
Mean
Standard deviation
Minimum and maximum values
RMS
Rolling statistics
Rate of change
Frequency-domain features
FFT-based vibration features
Time-window features
The system shall retain the selected features required by the deployed models.
Input:
Preprocessed sensor data.
Output:
Engineered feature set or feature sequences.
Priority: High

FR-AI-04 — Anomaly Detection
Description:
The system shall identify abnormal patterns in equipment sensor data.
The system shall use an appropriate trained machine learning or deep learning model to determine whether the current machine behavior is consistent with normal operating conditions.
The system shall generate an anomaly score and corresponding anomaly status.
Input:
Preprocessed sensor data and engineered features.
Output:
Anomaly score
Normal/Abnormal status
Machine identifier
Timestamp
Priority: Critical

FR-AI-05 — Remaining Useful Life Prediction
Description:
The system shall estimate the Remaining Useful Life (RUL) of monitored equipment based on historical and current sensor data.
The system shall support time-series models such as:
LSTM
GRU
1D-CNN
Other validated regression/time-series models
The selected model shall be determined based on experimental evaluation.
Input:
Historical and current sensor sequences.
Output:
Estimated RUL for the monitored machine.
Priority: Critical

FR-AI-06 — RUL Uncertainty Estimation
Description:
The system shall provide an uncertainty estimate or confidence range together with each RUL prediction.
The purpose is to communicate the uncertainty associated with the predicted remaining lifetime rather than presenting the prediction as an exact value.
Example:
Predicted RUL: 18 days
Estimated Range: 14–23 days
Input:
RUL model prediction.
Output:
RUL value and corresponding confidence/uncertainty range.
Priority: High

FR-AI-07 — Machine Health Severity Classification
Description:
The system shall classify the health condition of each monitored machine according to predefined severity levels.
The system shall support at least the following levels:
Info
Warning
Critical
The classification shall consider relevant AI/ML outputs such as anomaly scores, RUL predictions, and configured thresholds.
Input:
Anomaly score
RUL prediction
Configured thresholds
Output:
Machine health severity level.
Priority: Critical

FR-AI-08 — Explainable AI (XAI)
Description:
The system shall provide explanations for AI/ML predictions using Explainable AI techniques such as SHAP and LIME.
The explanation shall identify the most influential sensor features contributing to a prediction.
For example:
Critical machine condition detected.
Main contributing factors:
Increased vibration
High temperature
Abnormal pressure
The explanation shall be associated with the corresponding prediction.
Input:
Model prediction and input features.
Output:
Feature importance and prediction explanation.
Priority: Critical

FR-AI-09 — Configurable AI/ML Thresholds
Description:
The system shall allow authorized users to configure thresholds used for machine health assessment and alert classification.
The thresholds may include:
Anomaly score thresholds
RUL warning thresholds
Critical-condition thresholds
Sensor operating limits
Changes to thresholds shall be applied to subsequent predictions without requiring model retraining unless the model itself depends on the threshold.
Input:
Authorized threshold configuration.
Output:
Updated threshold configuration.
Priority: High

FR-AI-10 — AI/ML Model Inference
Description:
The system shall provide an inference service through the FastAPI backend to execute trained AI/ML models on new sensor data.
The inference service shall:
Receive sensor data.
Validate the input.
Apply the required preprocessing pipeline.
Execute the selected trained model.
Generate the required predictions.
Return the results in a structured format.
Output may include:
Anomaly score
Anomaly status
RUL
RUL uncertainty range
Severity level
XAI explanation
Priority: Critical

FR-AI-11 — Model Performance Evaluation
Description:
The system shall evaluate AI/ML models using appropriate performance metrics before deployment.
For classification and anomaly-detection tasks, the system shall support metrics including:
Precision
Recall
F1-score
ROC-AUC
Confusion Matrix
For RUL prediction and other regression tasks, the system shall support:
MAE
RMSE
The evaluation results shall be recorded and used to compare candidate models.
Input:
Test/validation data and model predictions.
Output:
Model performance metrics and evaluation results.
Priority: Critical

FR-AI-12 — Model Comparison and Selection
Description:
The system shall support comparison of multiple candidate AI/ML models based on predefined evaluation criteria.
Candidate models may include:
Random Forest
XGBoost
LightGBM
LSTM
GRU
1D-CNN
Autoencoders
The final model shall be selected based on its performance, reliability, explainability, and suitability for the intended task.
Input:
Candidate models and evaluation results.
Output:
Selected model and model comparison results.
Priority: High

FR-AI-13 — Model Versioning and Experiment Tracking
Description:
The system shall track AI/ML experiments and model versions using MLflow or an equivalent experiment-tracking system.
The system shall record, where applicable:
Model version
Model type
Training parameters
Dataset version
Evaluation metrics
Training date
Experiment identifier
This shall allow the team to identify which model version produced a specific prediction.
Input:
Training experiments and model artifacts.
Output:
Versioned and traceable model records.
Priority: High

FR-AI-14 — Data Drift Monitoring
Description:
The system shall monitor incoming sensor data for significant changes compared with the data distribution used during model training.
The system shall identify potential data drift that may affect model reliability.
When significant drift is detected, the system shall generate a notification or flag for ML model review.
Input:
New sensor data and reference training data.
Output:
Data drift status and drift notification when applicable.
Priority: Medium

FR-AI-15 — Model Retraining
Description:
The system shall support retraining of AI/ML models when required due to:
Significant data drift
Degraded model performance
Availability of new labeled failure data
Changes in equipment behavior
The retraining process shall produce a new model version while preserving the previous version for traceability.
Input:
Updated training dataset.
Output:
Retrained and versioned model.
Priority: Medium/High

FR-AI-16 — Prediction Logging
Description:
The system shall record AI/ML prediction results for monitoring, auditing, and future model evaluation.
Each prediction record should include, where applicable:
Machine ID
Timestamp
Input data reference
Model version
Anomaly score
RUL prediction
Severity level
XAI explanation reference
Priority: High

4. AI/ML Processing Flow
The overall AI/ML workflow shall follow this sequence:
Sensor Data
↓
Data Validation
↓
Time-Series Preprocessing
↓
Feature Engineering / FFT
↓
AI/ML Models
↓
Anomaly Detection + RUL Prediction
↓
Severity Classification
↓
XAI Explanation
↓
Prediction Results
↓
Logging & Monitoring

5. Functional Requirement Summary
ID
Requirement
Priority
FR-AI-01
Sensor Data Validation
High
FR-AI-02
Time-Series Data Preprocessing
High
FR-AI-03
Feature Engineering and Extraction
High
FR-AI-04
Anomaly Detection
Critical
FR-AI-05
Remaining Useful Life Prediction
Critical
FR-AI-06
RUL Uncertainty Estimation
High
FR-AI-07
Machine Health Severity Classification
Critical
FR-AI-08
Explainable AI using SHAP/LIME
Critical
FR-AI-09
Configurable AI/ML Thresholds
High
FR-AI-10
AI/ML Model Inference
Critical
FR-AI-11
Model Performance Evaluation
Critical
FR-AI-12
Model Comparison and Selection
High
FR-AI-13
Model Versioning and Experiment Tracking
High
FR-AI-14
Data Drift Monitoring
Medium
FR-AI-15
Model Retraining
Medium/High
FR-AI-16
Prediction Logging
High

6. Expected AI/ML Output
For each monitored machine, the AI/ML component should ultimately be capable of producing a structured result such as:
Machine ID: M-102
Anomaly Score: 0.87
Condition: Abnormal
RUL: 18 days
RUL Range: 14–23 days
Severity: Critical
Main Contributing Factors: High vibration, elevated temperature, abnormal pressure
Model Version: LSTM-v2.1
This output can then be consumed by the FastAPI backend, Streamlit dashboard, alert system, LLM Maintenance Assistant, and reporting module.